Subject:

Time for service and failure codes on glowplugs.

Vehicles concerned:

SAAB 9-5 (650)  My 10-- with Diesel Engine  A20DTH/LBS and A20DTR/LBY.

Background:

During certain driving conditions can an increased soot build up in the cylinder pressure sensor  block the pressure signal and cause the diagnostics to set fault codes.The pressure sensor is located in the glow plug.The typical failure codes are :P0399,P039C,P039D,P03A3,P03A6,P03A7,P03AD,P03B0,P03B1,P03B7,P03BA or P03BB. The failure code P1325 might  also be  present.

Description:

The driver gets message *Time for service* in DIC. No driveability issues.

Action:

Download latest ECM SW from TIS2WEB.

For the A20DTR/LBY there  is a new SW where the pressure sensor signal is not used. Engine calibration no: 12809663

For the A20DTH/LBS there is a new SW where the *Time for service* message is disabled. Engine calibration no: 12810697

Note!:

Notice :  Do not replace the glowplug or  crankshaft/camshaft sensors.Only a SPS is needed.

Warranty information:

Main object code: 34412

Failure: 67

Location: 05

Remedy: 05

Addtional objectcode: 10030

Stm time: 0,4t